User Manual

- Keypad on LCD

Select

- Instructions to run machine only

  - Plug-in power cord for machine.

  - Flip the switch on the back of the machine to turn it on

  - Take a look at the LCD at the front of the machine it should be lit with a white background and the word “Ready”

    - If not, flip the switch again.

  - Press the right arrow button, the display says, “>>Start”, press right arrow(or you can press the select button) again to start the machine

  - LCD displays the “Progress 0%” with a teal background

    - When the progress percentage reaches 100%, the machine resets to idle mode (“Ready”)

  - If you want to cancel the run while the machine is running, press and hold any key

- Instructions for priming pumps

  - With the machine on (displaying “Ready” on LCD), navigate through menu by pressing the down arrow until you see >>System Settings (“>>” means the menu item is selected, and if you want to open the submenu, press right arrow on keypad)

    - If you want to return to previous selection, press up arrow on keypad

  - Press right arrow on keypad

  - Then you should see >>Prime Pumps and press right arrow (this opens Prime Pump’s submenu)

  - Press down arrow to find and select the pump you want to prime

    - The machine only allows one pump to be primed at one time.

  - Then press right arrow (or press select button) on keypad to start priming the pump you selected

    - The LCD displays a message telling you which pump is being primed with a red background

  - The machine is going to continuously prime the pump until you press any key to stop the priming process

- Instructions for cleaning machine

  - Wipe down the machine to remove any residue from spills from normal use of the machine.

    - wipe the machine clean with a damp cloth and some soap, then use a dry cloth to remove any excess moisture.

  - To clean the lines and pumps of residue from liquids

    - Ensure stomach is clean. Place 1 cup of warm/hot water inside the stomach. Run 2 or 3 full cycles (The LCD should display “Ready”) press right key twice)

      - If the LCD is not displaying “Ready”, it must be at the Run state or priming, so press and hold any key to cancel)