Florida International University

School of Computing and Information Sciences

Software Engineering Focus

Final Deliverable

Automated Spherification Apparatus for Multisensory Molecular Gastronomy Experience

Team Members: Alex Collantes and Stephanie Cliett

Product Owner(s): Xavier Gonzalez

Mentor(s): Hadi Alhaffar, Research Associate

Instructor: Masoud Sadjadi

The MIT License (MIT)

Copyright (c) 2016 Florida International University

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE SOFTWARE.

Abstract

This document presents the information necessary to gain a good understanding of how this system works. The mission is to create a machine to automate the manual process of spherification of edible liquids. The manual process takes time to measure, mix, and create appropriate liquid mix. With this machine, the liquids are provided, the only thing needed is to let the machine do its job. This automated process takes 2.5 minutes instead 40 over minutes.

Our system focuses on improving the timing of mechanical components, enhancing the real estate of machine to allow future additions, upgrading current hardware, and rewriting code to be easily read and structured.

Table of Contents

Introduction ……………………………………………………………………………………………..... 5

Current System ……….…………………………………………………………………………………………………..5

Purpose of New System  …………………………………………………..……………………………. 5

User Stories

Implemented User Stories  ………………………………………….…………………………………..6

Pending User Stories  …………………………………………………………………...………………………………..….. 14

Project Plan

Hardware and Software Resources  ……………………………………………………………….… 16

Sprints Plan  ……………………………………………………………..………………………………………………………. 17

Sprint 1  …………………………………………………………………………………………………………………………... 17

Sprint 2  …………………………………………………………………………………………………………………………... 19

Sprint 3  …………………………………………………………………………………………………………………………... 20

Sprint 4  …………………………………………………………………………………………………………………………... 21

Sprint 5  …………………………………………………………………………………………………………………………... 22

Sprint 6  …………………………………………………………………………………………………………………………... 24

Sprint 7  …………………………………………………………………………………………………………………………... 25

System Design

Architectural Patterns  …………………………………………………………………………………………………..  27

System and Subsystem Decomposition  …………………………………………………………………………….…	28

Deployment Diagram  …………………………………………………………………………………………………….…...	28

Design Patterns  ………………………………………………………………………………………………………….…....	29

System Validation  …………………………………………………………………………………………………………………….	30

Glossary  ………………………………………………………………………………………………………………………………….	32

Appendix  ………………………………………………………………………………………………………………………………….	33

Appendix A - UML Diagrams  ……………………………………………………………………………………………….	33

Static UML Diagrams  ……………………………………………………………………………………………………….	33

Dynamic UML Diagrams  …………………………………………………………………………………………………..	35

Appendix B - User Interface Design  …………………………………………………………………………….…...	41

Appendix C - Sprint Review Reports  …………………………………………………………………………...……	43

Appendix D - User Manuals, Installation/Maintenance Document, Shortcomings/Wishlist Document and other documents  …………………………………………………………………….……………	51

References ……………………………………………………………………………………………………………………...………...	64

# Introduction

The Molecular Pearl Group (MPG) founded in 2016, created the first patented integrated device (Automated Spherification Apparatus for Multisensory Molecular Gastronomy Experience) that converts liquids into shelf-stable gel spheres (pearls) for use in myriad applications. Its patented technology targets the food/beverage industry providing a more efficient, consistent, and qualitatively improved product by mechanically automating the spherification process.

The manual spherification process requires the tedious steps of having to accurately weigh and mix ingredients at the right concentration for the pearls to form. Many times, the pearls are of uneven size, shape, and thickness. This project solves the productivity, labor cost, and quality problems with its automated dispensing machine and refill pack system. It only takes 2.5 minutes to consistently produce uniform pearls of any flavor with no gel residue.

## Current System

Automated Spherification Apparatus’s current system allows the user to navigate through a menu of options to select from. Whether it be to start the spherification process or do maintenance.  Currently, ingredients needed to mix in with the desired liquid the user wants to spherify, are included in the refill packs. But the user has to accurately measure the liquid (food liquid or alcohol) he/she wants to spherify before pouring into the machine. The machine produces a consistent geometry and taste for a limited amount of liquids. Also, waste is manually extracted.

## Purpose of New System

The purpose of the new implementation was to upgrade it with a volume sensor so that the user does not have to measure the desired liquid before pouring it into the machine. We also planned on adding a pH sensor so that the machine can automatically add buffer fluid when it detects false pH levels, which brings the machine closer to producing consistent geometry and taste for a variety of liquids. Another implementation was to automate extracting waste. But since COVID-19 interrupted our original goal, we focused on streamlining and improving the code, improve the timing of components to improve the shape of the spherical liquids, and enhancing the layout of the hardware in order to make it faster and easier for future engineers to add sensors.

# User Stories

The following section provides the detailed user stories that were implemented in this iteration of the Automated Spherification Apparatus for Multisensory Molecular Gastronomy Experience project. These user stories served as the basis for the implementation of the project’s features. This section also shows the user stories that are to be considered for future development.

## Implemented User Stories

# User Story Number: 19

# User Story Title: Structure code to be readable and easy to follow

- Description: As a developer, I would like to make a class to reflect functionality of the LCD menu so that, the original code is readable

- Acceptance Criteria:

  - Code compiles(passed)

  - Added methods to eliminate recycled code

  - Behavior is the same as before these code changes

  - Eliminate “freezing” effect

  - Machine should respond to button presses

Unit Test for User Story number 19

- Test case ID: Button response

- Description/Summary of test: when user is navigating through menu system, the machine responds to button presses

- Pre-condition: machine is on

- Expected Results:

  - LCD shows “Ready” on idle

  - Start Spherification process

    - User presses right button twice to start the spherification process

    - If user presses and holds any button, the machine stops the run

  - Navigate through menu system

    - Press right button once to enter the menu system

    - When user presses down button, it allows user to navigate through the menu system

    - When user presses left button, it allows user to go back to previous menu

    - When user presses up button, it allows user to go to previous selected item

- Status: (Pass/Fail): Pass

# User Story Number: 34

# User Story Title: Installed railing system to enhance layout

- Description: As a developer, I would like to install the railing system so that the electronic components can be mounted properly

- Acceptance Criteria:

  - Rail that has a sliding mechanism

  - Neatly position pumps to make more space in machine for future added hardware

  - Ensure it is sturdy

# User Story Number: 36

# User Story Title: Mount pump’s associated components to rail system

- Description: As a developer, I would like to mount the pump’s associated components to the railing system so that it is properly secured.

- Acceptance Criteria:

  - Ensure all tubing is connected

  - Ensure pump parts are connected properly

# User Story Number: 38

# User Story Title: Adjust Speed of new Stomach

- Description: As a developer, I would like to adjust the speed of the new stomach so that, it will match the specs

- Acceptance Criteria:

  - Match speed of old stomach

Unit test

- Test Case ID: Speed

- Description/Summary: Ensure the correct speed is set so that there would not be too many bubbles forming while mixing

- Pre-Condition: new stomach installed and electronically connected

- Expected Results: Not too many bubbles, so this is the correct speed

- Status (Pass/Fail): Pass

# User Story Number: 55

# User Story Title: Create User-Friendly Variables

- Description: As a developer, I would like to add new user-friendly variables for the start time of the different components, so that it is easier to change the values for the next team.

- Acceptance Criteria:

  - Easy way to change values of timings of different components of the machine

  - Positioned in one place so it is easy to access preferably, the top of code.

Unit test

- Test Case ID: Value reflects change

- Description/Summary: When changing the value of a variable, there is no need to scan the code to change this specific value

- Pre-Condition: no variable was assigned, the value was manually placed throughout the code

- Expected Results: No need to change this value throughout the code, you only have to change the value to once

# User Story Number: 62

# User Story Title: Adjust frequency of step motor

- Description: As a developer, I would like to adjust frequency of step motor, so that the pearls form properly

- Acceptance Criteria:

  - The stepping motor runs properly at the set frequency

  - Pearls form approximately round

Unit test

- Test Case ID: Approximately round

- Description/Summary: Adjust the frequency so that the pearls form approximately round

- Pre-Condition: Pearls had tails

- Expected Results: No tails

- Status (Pass/Fail): Pass

----------------------------------------------------------------------------------

# User Story Number: 29

# User Story Title: Create function to setup pumps

- Description: As a developer, I would like to create a method to initialize and configure the pumps, so that this section of code can be moved out of the main method

- Acceptance Criteria:

  - The method helps eliminate recycled/copied code

  - The method properly configures each pump

Unit test

- Test Case ID: T15-US29-SetupPumpsTest

- Description/Summary: Ensure that the pumps are working properly and set to the correct speed.

- Pre-Condition: Pumps are connected; machine is powered on.

- Expected Results: The pumps are observed to work correctly and at the proper speed. Changing speed variables noticeable affects the running speed of pumps.

- Status (Pass/Fail): Pass

# User Story Number: 32

# User Story Title: Debug code affecting step motor

- Description: As a developer, I would like to debug the code so that I can find the bug as to why the step motor is not working.

- Acceptance Criteria:

  - Step motor turns on

  - Step motor turns in the correct direction

  - Step motor runs at the correct speed

  - Quality of pearls is improved

Unit test

- Test Case ID: T15-US32-StepMotorTest

- Description/Summary: Ensure that the step motor turns on, turns in the correct direction, and runs at the correct speed.

- Pre-Condition: Step motor is connected; machine is powered on.

- Expected Results: The step motor is observed to work correctly, at the proper speed, and in the right direction. Changing frequency and direction variables noticeable affects the operation of the step motor. The quality of pearls has improved.

- Status (Pass/Fail): Pass

# User Story Number: 31

# User Story Title: Adjust step motor timing

- Description: As a developer, I would like to adjust the timing of the step motor, so that the pearls have the proper shape.

- Acceptance Criteria:

  - Pearls form approximately round

Unit test

- Test Case ID: T15-US31-StepMotorTimingTest

- Description/Summary: Ensure that the step motor properly pumps the mixture and pearls form approximately round

- Pre-Condition: Step motor is connected; machine is powered on.

- Expected Results: The step motor is observed to properly pump the mixture and create approximately round pearls.

- Status (Pass/Fail): Pass

# User Story Number: 12

# User Story Title: Restore electrical connections

- Description: As a developer, I would like to restore the original electrical connections of the prototype, so that the Arduino is able to control the pumps.

- Acceptance Criteria:

  - When connecting each component, validate which pin to connect to by using past pictures

  - Validate where to orient each component from past pictures

Unit test

- Test Case ID: T15-US12-ElectricalConnectionsTest

- Description/Summary: Ensure that the components are connected to the right pins.

- Pre-Condition: All boards and pumps connected; machine is powered on.

- Expected Results: All components are observed to work properly when running the machine.

- Status (Pass/Fail): Pass

# User Story Number: 14

# User Story Title: Restore liquid mixer

- Description: As a developer, I would like to restore the liquid mixer (stomach), which mixes the liquids to perform the chemical reaction.

- Acceptance Criteria:

  - Connect wire to correct pin

  - Orient the mixer to the correct position

  - Route appropriate pipes into the mixer

Unit test

- Test Case ID: T15-US14-LiquidMixerTest

- Description/Summary: Ensure that the liquid mixer is connected to the right pin.

- Pre-Condition: Liquid mixer connected; machine is powered on.

- Expected Results: Liquid mixer is observed to work properly when running the machine.

- Status (Pass/Fail): Pass

# User Story Number: 35

# User Story Title: Mount pumps

- Description: As a developer, I would like to mount all pumps on the railing system so that they are secured and streamlined.

- Acceptance Criteria:

  - Pumps should fit snuggly into 3D printed parts so there is no wobble/movement when operating the machine.

  - 3D parts should be able to slide into railing system.

Unit test

- Test Case ID: T15-US35-MountPumpsTest

- Description/Summary: Ensure that the pumps are secure.

- Pre-Condition: Pumps mounted on the railing system; machine is powered on.

- Expected Results: The pumps are observed to be secure when running the machine.

- Status (Pass/Fail): Pass

# User Story Number: 37

# User Story Title: Replace blender

- Description: As a developer, I would like to replace the current blender/stomach with the new upgraded version, so that more liquid can be mixed.

- Acceptance Criteria:

  - Blender should turn on at 5 seconds when machine is running the spherification process.

  - Blender should be oriented in same place original blender was.

Unit test

- Test Case ID: T15-US37-ReplaceBlenderTest

- Description/Summary: Ensure that the new blender works properly and is placed correctly.

- Pre-Condition: New blender installed; machine is powered on.

- Expected Results: The blender is observed to work properly when running the machine.

- Status (Pass/Fail): Pass

# User Story Number: 58

# User Story Title:  Design blender lid

- Description: As a developer, I would like to design the lid for the blender/stomach, so that each tubing will be channeled properly into the stomach.

- Acceptance Criteria:

  - Channel the appropriate tubing into the stomach

  - Ensure tubing is not in the way of the blade

  - Ensure tubing is secured in place

Unit test

- Test Case ID: T15-US58-DesignLidTest

- Description/Summary: Ensure that the lid securely channels all of the necessary tubing and keeps tubing from hitting the blender’s blade.

- Pre-Condition: New lid installed on blender; machine is powered on.

- Expected Results: The blender’s new lid is observed to properly channel all of the necessary tubing and keeps tubing from hitting the blender’s blade when running the machine.

- Status (Pass/Fail): Pass

# User Story Number: 61

# User Story Title:  Install braces

- Description: As a developer, I would like to install hardware to route the tubing, so that the layout design is neat.

- Acceptance Criteria:

  - Braces that hold tubing are properly attached to the machine body

  - Braces are correctly placed to help improve the layout of the machine

Unit test

- Test Case ID: T15-US61-BracesTest

- Description/Summary: Ensure that the braces properly help route tubing, hold tubing in place, and help improve the layout of the machine.

- Pre-Condition: Braces installed inside machine; machine is powered on.

- Expected Results: The braces are observed to hold tubing securely in place and improve the layout of the machine.

- Status (Pass/Fail): Pass

# User Story Number: 56

# User Story Title:  Configure timing of components

- Description: As a developer, I would like to configure the timing of each component of system, so that I can ensure desired results of pearls.

- Acceptance Criteria:

  - Ensure the timing of each component starts and finishes at time given by PO

  - Pearls should form approximately round

Unit test

- Test Case ID: T15-US56-ComponentsTimingTest

- Description/Summary: Ensure that the variables in the source code are set to the correct values so that the timings of all components inside the machine are as expected by the PO and the resulting pearls have the desired shape.

- Pre-Condition: Source code has been updated and uploaded into the machine; machine is powered on.

- Expected Results: All components are observed to behave as expected by the PO and the resulting pearls are approximately round.

- Status (Pass/Fail): Pass

## Pending User Stories

User Story Number 23

- Description: As a developer, I would like to do research on the parts needed for the volume sensor that is compatible, so that I can integrate it

User Story Number 24

- Description: As a developer, I would like to study the library for the particular volume sensor selected, so that I know how to program the sensor.

User Story number 25

- Description: As a developer, I would like to code the volume sensor, so that I can implement functionality needed for the different tubings in this machine.

User Story number 26

- Description: As a developer, I would like to install the volume sensor, so that I can test my code

User Story number 40

- Description: As a developer, I would like to do research for the appropriate pump to extract the calcium solution, so that it is compatible with the components currently in the machine.

User Story number 41

- Description: As developer, I would like to install the pump to extract the calcium pump, so that it can automatically extract the calcium solution instead of the user

User Story number 42

- Description: As a developer, I would like to code the pump, so that it extracts the calcium solution at the correct timing.

User Story number 43

- Description: As a user, I would like the machine to extract the calcium solution by a press of a button, so that I do not have to do it myself

User Story number 5

- Description: As a user, I'd like the device to be able to detect the pH levels of different liquids so that each liquid has the correct amount of pH.

User Story Number 6

- Description: As a user, I'd like the device to calculate the appropriate amount of substances needed for spherification using the pH levels of a wide variety of drinks, sauces, liquid medicine, etc.

User Story number 8

- Description: As a developer, I will read documentation of pH sensor purchased in order to attach the sensor properly onto the Arduino so that it will accept the power to turn on.

# Project Plan

## Hardware and Software Resources

The following is a list of all hardware and software resources that were used in this project:

Hardware:

OS: Windows, Mac, or Linux

Memory: 392.2 MB to download Arduino IDE, at least 4 GB of RAM

Arduino Board is available, no need to get one.

Other hardware to be added to machine will need to be food gradable

Software:

Arduino IDE-write program to control Arduino Board

Project Management and software development tools:

Trello- To manage tasks (this is similar to a Kanban Board)

GitHub-Manage version control

Google Drive-cloud share storage. This helps when sharing and editing files like Word, Excel, or PowerPoints with your team

## Sprints Plan

### Sprint 1

Sprint Planning Meeting Minutes:

Date: 1/14/2020

Start Time: 12:00 pm

End Time: 3:00 pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier González, Hadi Alhaffar, Argelio Maldonado

After discussion, the velocity of the team was estimated to be 40 hours

The product owner chose the following user stories to be done during the next sprint. The order is based on priority

- User Story 1 Meet up with the engineer that designed and programmed the spherification device.

- User Story 2 Learn how to wire Arduino hardware

- User Story 3 Learn how to program Arduino microcontrollers

- User Story 3a Learn Arduino architecture

- User Story 3b Setting up Development Environment

- User Story 3c Learn how to use Arduino IDE

- User Story 7 Take a look at machine components for possible enhancements

- User Story 4 Research pH sensor compatible with Arduino board

The team members indicated thier willingness to work on the following user stories

- Alex Collantes

  - User Story 1 Meet up with the engineer that designed and programmed the spherification device.

  - User Story 2 Learn how to wire Arduino hardware

  - User Story 3 Learn how to program Arduino microcontrollers

  - User Story 3a Learn Arduino architecture

  - User Story 3b Setting up Development Environment

  - User Story 3c Learn how to use Arduino IDE

  - User Story 7 Take a look at machine components for possible enhancements

  - User Story 4 Research pH sensor compatible with Arduino board

- Stephanie Cliett

  - User Story 1 Meet up with the engineer that designed and programmed the spherification device.

  - User Story 2 Learn how to wire Arduino hardware

  - User Story 3 Learn how to program Arduino microcontrollers

  - User Story 3a Learn Arduino architecture

  - User Story 3b Setting up Development Environment

  - User Story 3c Learn how to use Arduino IDE

  - User Story 7 Take a look at machine components for possible enhancements

  - User Story 4 Research pH sensor compatible with Arduino board

### Sprint 2

Sprint Planning Meeting Minutes:

Date: 1/29/2020

Start Time: 6:45 pm

End Time: 7:00 pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier González, Hadi Alhaffar

After discussion, the velocity of the team was estimated to be 40 hours

The product owner chose the following user stories to be done during the next sprint. The order is based on priority

- User Story 12 Restore original electrical connections of current prototype

- User Story 13 Restore pipe connections

- User Story 14 Restore liquid mixer (stomach)

- User Story 15 Test prototype after restoration

- User Story 16 Troubleshoot step motor malfunction

- User Story 17 Validate the connection to the stepper is correct

The team members indicated their willingness to work on the following user stories

- Alex Collantes

  - User Story 12 Restore original electrical connections of current prototype

  - User Story 14 Restore liquid mixer

  - User Story 16 Troubleshoot step motor malfunction

- Stephanie Cliett

  - User Story 13 Restore pipe connection

  - User Story 15 Test prototype after restoration

  - User Story 17-Validate the connection to the stepper is correct

### Sprint 3

Sprint Planning Meeting Minutes:

Date: 2/11/2020

Start Time: 5:30pm

End Time: 6:00pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier González

After discussion, the velocity of the team was estimated to be 40 hours

The product owner chose the following user stories to be done during the next sprint. The order is based on priority

- User Story 18 Clean up the code from unused libraries

- User Story 28 Clean up the code from unused variables

- User Story 29 Create a method to initialize and configure pumps

- User Story 22 Research on volume sensors that is compatible

- User Story 19 Create an LCD menu class in order to give structure to code

- User Story 20 Add comments to methods created

- User Story 21 Add comments to variables

The team members indicated their willingness to work on the following user stories

- Alex Collantes

  - User Story 28 Clean up the code from unused variables

  - User Story 29 Create a method to initialize and configure pumps

  - User Story 22 Research on volume sensors that is compatible

  - User Story 20 Add comments to methods created

  - User Story 21 Add comments to variables

- Stephanie Cliett

  - User Story 18 Clean up the code from unused libraries

  - User Story 19 Create an LCD menu class in order to give structure to code

  - User Story 20 Add comments to methods created

  - User Story 21 Add comments to variables

### Sprint 4

Sprint Planning Meeting Minutes:

Date: 3/02/2020

Start Time: 8:11 pm

End Time: 8:41 pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier González

After discussion, the velocity of the team was estimated to be 40 hours

The product owner chose the following user stories to be done during the next sprint. The order is based on priority

- User Story 30 Examine the step motor for obstruction

- User Story 19 Create an LCD menu class in order to give structure to code

- User Story 32 Debug code to troubleshoot why the step motor is not turning on

- User Story 33 Research suitable railing system

- User Story 34 Install railing system

- User Story 35 Mount all pumps onto railing system

- User Story 36 Mount pump’s associated components onto railing system

- User Story 37 Replace current stomach with a new upgraded version

- User Story 38 Adjust the speed of the new stomach

- User Story 39 Adjust code with new stomach

- User Story 27 Start on use case diagram

The team members indicated their willingness to work on the following user stories

- Alex Collantes

  - User Story 32 Debug code to troubleshoot why the step motor is not turning on

  - User Story 33 Research suitable railing system

  - User Story 35 Mount all pumps onto railing system

  - User Story 37 Replace current stomach with a new upgraded version

- Stephanie

  - User Story 30 Examine the step motor for obstruction

  - User Story 19 Create an LCD menu class in order to give structure to code

  - User Story 34 Install railing system

  - User Story 36 Mount pump’s associated components onto railing system

  - User Story 38 Adjust the speed of the new stomach

  - User Story 39 Adjust code with new stomach

  - User Story 27 Start on use case diagram

### Sprint 5

Sprint Planning Meeting Minutes:

Date: 3/17/2020

Start Time: 8:30 pm

End Time: 9:00 pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier González

After discussion, the velocity of the team was estimated to be 40 hours

The product owner chose the following user stories to be done during the next sprint. The order is based on priority

- User Story 44 Research for main issues for step motor not operating correctly

- User Story 45 Research on physical components of the step motor to identify damage

- User Story 46 Meet with engineer to get advice on fixing timings

- User Story 31 Adjust timing of step motor

- User Story 19b test the methods I created in LCD class

- User Story 47 Document research on main issues of step motor not operating correctly

- User Story 48 Document research on physical components of the step motor to identify damage

- User Story 49 Pick up the machine from school b/c of lockdown

- User Story 30 troubleshoot step motor

- User Story 51 Migrate code for the step motor into its own method

- User Story 50 Troubleshoot “jump” of the step motor

- User Story 53 Research upgrading tubing

- User Story 54 Replace current tubing

- User Story 19c Debug code for button response

The team members indicated their willingness to work on the following user stories

- Alex Collantes

  - User Story 46 Meet with engineer to get advice on fixing timings

  - User Story 31 Adjust timing of step motor

  - User Story 49 Pick up the machine from school b/c of lockdown

  - User Story 30 Troubleshoot the step motor after cleaning

  - User Story 51 Migrate code for the step motor into its own method

  - User Story 50 Troubleshoot “jump” of the step motor

  - User Story 53 Research upgrading tubing

  - User Story 54 Replace current tubing

- Stephanie Cliett

  - User Story 44 Research for main issues for step motor not operating correctly

  - User Story 45 Research on physical components of the step motor to identify damage

  - User Story 47 Document research on main issues of step motor not operating correctly

  - User Story 48 Document research on physical components of the step motor to identify damage

  - User Story 19b test the methods I created in LCD class

  - User Story 19c Debug code for button response

  - User Story 31 Adjust timing of step motor

### Sprint 6

Sprint Planning Meeting Minutes:

Date: 3/30/2020

Start Time: 7:00 pm

End Time:  7:30 pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier González

After discussion, the velocity of the team was estimated to be 40 hours

The product owner chose the following user stories to be done during the next sprint. The order is based on priority

- User Story 58 Label the tubing of pumps

- User Story 59 Design the lid so that the tubing can be channeled through

- User Story 60 Design a diagram to show the layout of components

- User Story 61 Install hardware to route the tubing

- User Story 31 validate the timing of stepping motor

- User Story 62 Adjust frequency of step motor for pearls to form properly

- User Story 56 Configure timing of each component

- User Story 39 Adjust code of new stomach for pearls to form properly

- User Story 57 Create a class diagram

The team members indicated their willingness to work on the following user stories

- Alex Collantes

  - User Story 58 Label the tubing of pumps

  - User Story 59 Design the lid so that the tubings can be channeled through

  - User Story 61 Install hardware to route the tubing

  - User Story 31 validate the timing of stepping motor

  - User Story 56 Configure timing of each component

- Stephanie Cliett

  - User Story 60 Design a diagram to show the layout of components

  - User Story 31 validate the timing of stepping motor

  - User Story 62 Adjust frequency of step motor for pearls to form properly

  - User Story 39 Adjust code of new stomach for pearls to form properly

  - User Story 57 Create a class diagram

### Sprint 7

Sprint Planning Meeting Minutes:

Date: 4/13/2020

Start Time: 7:00 pm

End Time:  7:30 pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier González

After discussion, the velocity of the team was estimated to be 40 hours

The product owner chose the following user stories to be done during the next sprint. The order is based on priority

- User Story 64 Finish final touches to the code

- User Story 65 Create a README.txt file

- User Story 66 Create an Installation/Maintenance Guide

- User Story 67 Create a User Manual

- User Story 68 Create a Daily Scrum Meeting file

- User Story 69 Create a Daily Scrum Meeting file

- User Story 70 Create a Shortcomings/Wishlist file

- User Story 71 Create a Documents directory to hold all documents

- User Story 72 Create a poster to present the project problem, solution, and implementation

- User Story 73 Create a poster to present the project problem, solution, and implementation

- User Story 86 Create presentation slides that show accomplishments made on the project

- User Story 87 Create presentation slides that show accomplishments made on the project

- User Story 75 Create a video that introduces the software solution

- User Story 76 Create a video on how to use the software solution (each actor)

- User Story 77 Create a video that shows how to install and maintain the software solution

- User Story 78 Create a video that shows what the shortcomings of the current software and what features are missing

- User Story 79 Add videos to the professor’s account

- User Story 80 Create an index.html file that list the four videos that direct the user to YouTube to watch each of the videos

- User Story 81 Create a final documentation file that contains the necessary documents for the next team to read to get familiar with the project

The team members indicated their willingness to work on the following user stories

- Alex Collantes

  - User Story 64 Finish final touches to the code

  - User Story 66 Create an Installation/Maintenance Guide

  - User Story 67 Create a User Manual

  - User Story 69 Create a Daily Scrum Meeting file

  - User Story 70 Create a Shortcomings/Wishlist file

  - User Story 72 Create a poster to present the project problem, solution, and implementation

  - User Story 87 Create presentation slides that show accomplishments made on the project

  - User Story 77 Create a video that shows how to install and maintain the software solution

  - User Story 78 Create a video that shows what the shortcomings of the current software and what features are missing

  - User Story 79 Add videos to the professor’s account

  - User Story 80 Create an index.html file that list the four videos that direct the user to YouTube to watch each of the videos

- Stephanie Cliett

  - User Story 65 Create a README.txt file

  - User Story 67 Create a User Manual

  - User Story 68 Create a Daily Scrum Meeting file

  - User Story 71 Create a Documents directory to hold all documents

  - User Story 73 Create a poster to present the project problem, solution, and implementation

  - User Story 88 Create presentation slides that show accomplishments made on the project

  - User Story 75 Create a video that introduces the software solution

  - User Story 76 Create a video on how to use the software solution (each actor)

  - User Story 81 Create a final documentation file that contains the necessary documents for the next team to read to get familiar with the project

# System Design

## Architectural Patterns

Model-View-Controller (MVC) pattern was selected for this project as shown below:

- Model — contains the core functionality

  - Arduino Uno board

- View — displays the information to the user

  - LCD Screen

- Controller — handles the input from the user

  - Input Interface on LCD board

## System and Subsystem Decomposition

## Deployment Diagram

The source code for the software is directly uploaded to the Arduino Uno board. No deployment is needed for this hardware-based project.

## Design Patterns

State pattern was selected for this project. This pattern allows an object to alter its behavior when its internal state changes. The state pattern is set to solve two main problems:

- An object should change its behavior when its internal state changes.

- State-specific behavior should be defined independently. That is, adding new states should not affect the behavior of existing states.

# System Validation

Test Suite ID 1

Purpose: Test functionality of use case Start Spherification

Test case 1:

- Purpose: Ensure menu system works properly and machine runs spherification process fully

- Pre-Condition: Machine is turned on

- Input: User navigates through menu and selects start

- Expected Results: User is able to navigate to start by pressing right button and start the spherification process by pressing right button again

- Actual Result: User is able to select start and machine starts spherification process

- Status (Fail/Pass): Pass

Test Case 2:

- Purpose: Ensure pearls are approximately round

- Pre-Condition: Spherification already started by user

- Expected Results: Pearls are formed without tails

- Actual Result: Pearls formed without tails

- Status (Pass/Fail): Pass

Test Suite ID 2

Purpose: Test functionality of use case prime pumps

Test Case 1:

- Purpose: Ensure meu system works properly and machine runs prime pumps

- Pre-Condition: Machine is on

- Input: User navigates through menu and selects prime pumps

- Expected Results: User is able to navigate to the prime pumps menu and is able to run prime pump maintenance

- Actual Result: User navigated to the prime pumps menu and selected the pump he/she wants to prime for maintenance

- Status: (Pass/Fail): Pass

Test Case 2:

- Purpose: Ensure when the user requests to stop the prime pump maintenance, the pump stops priming

- Pre-Condition: pump is priming

- Input: User presses any key

- Expected Results: After the user presses any key, the pump stops priming

- Actual Results: After the user pressed any key, the pump stopped priming

- Status (Pass/Fail): Pass

# Glossary

- Spherification - is a modern cuisine technique that involves creating semi-solid spheres with thin membranes out of liquids

# Appendix

## Appendix A

## Static UML Diagram

Alex Collantes:

Stephanie:

## Dynamic UML Diagrams

Alex Collantes:

Use Case Diagram

## Sequence Diagrams

## Stephanie Cliett:

## Use Case Diagram

## Sequence Diagrams

## Appendix B - User Interface Design

## Appendix C - Sprint Review Reports

### Sprint 1

Sprint Review Meeting Minutes:

Date: 1/24/2020

Start Time: 4:00pm

End Time:  4:20pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier Gonzalez

User stories accepted by the product owner:

- User Story 7: Take a look at machine components for possible enhancements

- User Story 3a: Learn Arduino architecture

- User Story 3b: Setting up Development Environment

- User Story 3c: Learn how to use Arduino IDE

- User Story 2: Learn how to wire Arduino hardware

- User Story 3: Learn how to program Arduino microcontrollers

- User Story 7: Take a look at machine components for possible enhancements

- User story 4: Research pH sensor compatible with Arduino board

User stories rejected by the product owner:

- None

### Sprint 2

Sprint Review Meeting Minutes:

Date: 02/10/2020

Start Time: 7:00pm

End Time:  7:20pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier Gonzalez

User stories accepted by the product owner:

- User Story 12: Restore original electrical connections of current prototype

- User Story 13: Restore pipe connections

- User Story 14: Restore liquid mixer (stomach)

- User Story 15: Test prototype after restoration

- User Story 9: Learn how the machine works

- User Story 10: Identify unnecessary components inside machine

- User Story 11: Identify improvements to the original source code

- User Story 16: Troubleshoot step motor malfunction

- User Story 17: Validate the connection to the stepper is correct

User stories rejected by the product owner:

- None

### Sprint 3

Sprint Review Meeting Minutes:

Date: 02/21/2020

Start Time: 2:00pm

End Time:  2:30pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier Gonzalez

User stories accepted by the product owner:

- User Story 18: Clean up the code from unused libraries

- User Story 28: Clean up the code from unused variables

- User Story 21: Add comments to variables

- User Story 20: Add comments to methods created

- User Story 22: Research on volume sensors that is compatible

- User Story 29: Create a method to initialize and configure pumps

User stories rejected by the product owner:

- User Story 19: Create an LCD menu class in order to give structure to code

### Sprint 4

Sprint Review Meeting Minutes:

Date: 03/13/2020

Start Time: 6:00pm

End Time:  6:30pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier Gonzalez

User stories accepted by the product owner:

- User Story 32: Debug code to troubleshoot why the step motor is not turning on

- User Story 30: Examine the step motor for obstruction

- User Story 27: Start on use case diagram

- User Story 33: Research suitable railing system

- User Story 34: Install railing system

- User Story 35: Mount all pumps onto railing system

- User Story 36: Mount pump’s associated components onto railing system

- User Story 37: Replace current stomach with a new upgraded version

- User Story 38: Adjust the speed of the new stomach

User stories rejected by the product owner:

- User Story 19b: Validate methods in LCD class

### Sprint 5

Sprint Review Meeting Minutes:

Date: 03/27/2020

Start Time: 7:00pm

End Time:  7:50pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier Gonzalez

User stories accepted by the product owner:

- User Story 44: Research for main issues for step motor not operating correctly

- User Story 47: Document research on main issues of step motor not operating correctly

- User Story 45: Research on physical components of the step motor to identify damage

- User Story 48: Document research on physical components of the step motor to identify damage

- User Story 46: Meet with engineer to get advice on fixing timings

- User Story 49: Pick up the machine from school b/c of lockdown

- User Story 30: Troubleshoot step motor

- User Story 30b: Test step motor after cleaning it

- User Story 51: Migrate code for the step motor into its own method

- User Story 50: Troubleshoot “jump” of the step motor

- User Story 53: Research upgrading tubing

- User Story 54: Replace current tubing

- User Story 19b: Test the methods I created in LCD class

- User Story 19c: Debug code for button response

User stories rejected by the product owner:

- None

### Sprint 6

Sprint Review Meeting Minutes:

Date: 04/12/2020

Start Time: 3:30pm

End Time:  4:00pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier Gonzalez

User stories accepted by the product owner:

- User Story 58: Label the tubing of pumps

- User Story 59: Design the lid so that the tubing can be channeled through

- User Story 60: Design a diagram to show the layout of components

- User Story 63: Design a diagram of the different boards used

- User Story 61: Install hardware to route the tubing

- User Story 55: Add new variables for the start time of the different components

- User Story 62: Adjust frequency of step motor for pearls to form properly

- User Story 31: Validate the timing of stepping motor

- User Story 39: Adjust code of new stomach for pearls to form properly

- User Story 57: Create a class diagram

User stories rejected by the product owner:

- None

### Sprint 7

Sprint Review Meeting Minutes:

Date: 04/25/2020

Start Time: 4:30pm

End Time:  5:00pm

Attendees: Stephanie Cliett, Alex Collantes, Xavier Gonzalez

User stories accepted by the product owner:

- User Story 64: Finish final touches to the code

- User Story 65: Create a README.txt file

- User Story 66: Create an Installation/Maintenance Guide

- User Story 67: Create a User Manual

- User Story 68: Create a Daily Scrum Meeting file

- User Story 69: Create a Daily Scrum Meeting file

- User Story 70: Create a Shortcomings/Wishlist file

- User Story 71: Create a Documents directory to hold all documents

- User Story 72: Create a poster to present the project problem, solution, and implementation

- User Story 73: Create a poster to present the project problem, solution, and implementation

- User Story 82: Summarize the problem solved in this project

- User Story 84: Add diagrams to the presentation slides

- User Story 85: Add system and architecture information to the presentation slides

- User Story 86: Create presentation slides that show accomplishments made on the project

- User Story 87: Create presentation slides that show accomplishments made on the project

- User Story 83: Add Gantt chart to the presentation slides

- User Story 75: Create a video that introduces the software solution

- User Story 76: Create a video on how to use the software solution (each actor)

- User Story 77: Create a video that shows how to install and maintain the software solution

- User Story 78: Create a video that shows what the shortcomings of the current software and what features are missing

- User Story 79: Add videos to the professor’s account

- User Story 80: Create an index.html file that list the four videos that direct the user to YouTube to watch each of the videos

- User Story 81: Create a final documentation file that contains the necessary documents for the next team to read to get familiar with the project

- User Story 81b: Write an introduction for the final documentation

- User Story 88: Create presentation slides that show accomplishments made on the project

- User Story 89: Add project plan information to final deliverable

- User Story 90: Add system design information to final deliverable

- User Story 91: Add system validation information to final deliverable

- User Story 92: Complete glossary section in final deliverable

- User Story 72: Create poster summarizing the project

- User Story 73: Create poster summarizing the project

User stories rejected by the product owner:

- None

## Appendix D - User Manuals, Installation/Maintenance Document, Shortcomings/Wishlist Document and other documents

# Installation / Maintenance Guide

Installation

Step 1 – Downloading source code files

- Click this link to download the source code from the GitHub repository: https://github.com/SeniorProject2020AS/SeniorProject2020AS-repo/blob/master/Machine.zip

- Click on the “Download” button

- Save the file

Step 2 – Extracting source code files

- Right-click on the file named “Machine.zip” and select “Extract All…”

- Select a location for your extracted files and click “Extract”

- Browse inside the “Machine” folder and confirm that it contains 3 files as shown below:

Step 3 – Downloading and Installing the Arduino IDE

- Click this link to download the Arduino IDE from the official Arduino downloads page: https://www.arduino.cc/en/Main/Software

- Download the Arduino IDE by selecting your Operating System from the options shown to the right

- Save and open the installation file

- Complete the installation by following the on-screen instructions

Step 4 – Configuring the Arduino IDE

- Open the “Machine.ino” file from the source code files extracted in Step 2

- On the menu bar at the top, click on Sketch > Include Library > Manage Libraries…

- In the search bar, type “Adafruit Motor Shield V2” and install the first option from the search results (do not close the window when finished)

- In the search bar, clear the previous search and type “Adafruit RGB LCD Shield”, then install the first option from the search results and close the window

- Click on the “Verify” button (Checkmark) on the top-left corner to verify that all necessary libraries are installed

- Confirm that the verification of the source code and libraries was successful by checking that the status bar at the bottom (green-colored) reads “Done compiling.”

- Connect the USB cable attached to the Arduino Uno board inside machine to any USB port on your computer

- On the menu bar at the top, confirm that the “Arduino Uno” board is selected under Tools > Board: “Arduino Uno” > Arduino Uno

- On the menu bar at the top, select the COM port labeled “Arduino Uno” under Tools > Port > COM4 (Arduino Uno) (the port number may change, in this case it is “COM4”)

Step 5 – Uploading code to the Arduino Uno board

- Connect the USB cable attached to the Arduino Uno board inside the machine to any USB port on your computer

- Click on the “Upload” button (Arrow) on the top-left corner to compile and upload the source code to the Arduino Uno board inside the machine (this will overwrite the program currently being executed by the Arduino Uno board)

  - It is only necessary to upload after making code changes

- Verify that the source code was successfully uploaded by checking that the status bar at the bottom is green-colored and reads “Done uploading.”

Maintenance

Step 1 – Version Control

- When making any maintenance changes to the source code, use GitHub, Bitbucket, Azure DevOps, or any other similar version control software tool to ensure that all modifications to the source code are recorded and can be reverted if necessary.

- When recording any changes to the source code on your local repository of the version control software, click on the built-in “Verify” button (Checkmark) on the top-left corner of the Arduino IDE to ensure that the source code does not contain any syntax errors.

Step 2 – General Debugging

- The file named “Machine.ino” has a small section at the top where all variables that affect the shape of the pearls, resulting from running a full cycle of the machine, have been isolated from the remainder of the code. These values can be modified to help troubleshoot issues affecting the formation/geometry of the pearls.

# User Manual

- Keypad on LCD

Select

Previous selection

Open submenu/start

Back to previous menu

navigate

- Instructions to run machine only

  - Plug-in power cord for machine.

  - Flip the switch on the back of the machine to turn it on

  - Take a look at the LCD at the front of the machine it should be lit with a white background and the word “Ready”

    - If not, flip the switch again.

  - Press the right arrow button, the display says, “>>Start”, press right arrow again to start the machine (or you can press the select button)

  - LCD displays the “Progress 0%” with a teal background

    - When the progress percentage reaches 100%, the machine resets to idle mode (“Ready”)

  - If you want to cancel the run while the machine is running, press and hold any key

- Instructions for priming pumps

  - With the machine on (displaying “Ready” on LCD), navigate through menu by pressing the down arrow until you see >>System Settings (“>>” means the menu item is selected, and if you want to open the submenu, press right arrow on keypad)

    - If you want to return to previous selection, press up arrow on keypad

  - Press right arrow on keypad

  - Then you should see >>Prime Pumps and press right arrow (this opens Prime Pump’s submenu)

  - Press down arrow to find and select the pump you want to prime

    - The machine only allows one pump to be primed at one time.

  - Then press right arrow (or select button) on keypad to start priming the pump you selected

    - The LCD displays a message telling you which pump is being primed with a red background

  - The machine is going to continuously prime the pump until you press any key to stop the priming process

- Instructions for cleaning machine

  - Wipe down the machine to remove any residue from spills from normal use of the machine.

    - wipe the machine clean with a damp cloth and some soap, then use a dry cloth to remove any excess moisture.

  - To clean the lines and pumps of residue from liquids

    - Ensure stomach is clean. Place 1 cup of warm/hot water inside the stomach. Run 2 or 3 full cycles (The LCD should display “Ready”) press right key twice)

      - If the LCD is not displaying “Ready”, it must be at the Run state or priming, so press and hold any key to cancel)

# Shortcomings/Wishlist

- Implement the use of hardware interrupts, which are supported by the Arduino microcontroller, to eliminate the need to query input devices and improve the overall performance of the machine.

  - This link shows you how to implement an interrupt in Arduino

    - https://www.arduino.cc/reference/en/language/functions/external-interrupts/attachinterrupt/

- Further improve the operating frequency of the stepping motor, so that the resulting pearls will be perfectly spherical instead of teardrop shaped.

- Further improve the timing of all components in the machine, so that the process of creating pearls is optimized.

- Install and configure volume sensors to automatically detect how much volume of liquids has been input by the user and help measure the volume of other chemicals needed to perform a successful chemical reaction, which results in perfectly spherical pearls.

  - This link shows you the volume sensor we had in mind:

    - https://www.dfrobot.com/product-1531.html?gclid=EAIaIQobChMIjevK8pHc5wIVg5-fCh27tgPvEAkYDyABEgKKu_D_BwE

- Install an additional pump, not necessarily peristaltic, that will extract the contaminated bath solution and dispose of it.

  - This link shows you the pump we had in mind

    - https://www.harborfreight.com/12v-dc-transfer-pump-290-gph-63324.html

- Modify the machine so that the cartridges holding the chemicals used to perform the chemical reaction can be attached and removed externally, thus eliminating the need to open the machine to perform this task.

- Upgrade the LCD display

  - This link shows you the LCD display we had in mind:

    - https://create.arduino.cc/projecthub/electropeak/ultimate-beginner-s-guide-to-run-tft-lcd-displays-by-arduino-081006

- Allow the user to select the liquid he/she wants to produce instead of pouring the liquid his/herself

- Install and configure pH sensors that will automatically detect the pH levels of liquids input by the user and help measure the volume of chemicals needed to regulate the pH of the mixture, that way perform a successful chemical reaction, which results in perfectly spherical pearls.

  - This link is an example of a pH sensor we found:

    - https://www.dfrobot.com/product-1025.html

# References

- Arduino website to download IDE

  - https://www.arduino.cc/en/main/software

- Arduino tutorials

  - Paul McWhorter has good video tutorials for beginners

    - https://www.youtube.com/user/mcwhorpj/featured

- We used the websites below to order supplies

  - https://www.mcmaster.com/

  - https://www.amazon.com/